Water

Open

If you are starting the water in the winter, warm the cabin above 60 and open cabinet doors under kitchen and bathroom sinks to thaw valves before you operate them. The hot water heater drain valve may require heating with a hot air hair dryer. The outside valves are heat taped and should operate after a few minutes with the electricity on.

- Close all water valves in the cabin except the bathroom sink hot water faucet on left side of sink.

  - Blue valve under kitchen sink

  - Kitchen sink faucet

  - Cold water faucet in bathroom sink

  - Shower ON and OFF handle

  - Hot water tank drain valve (gray plastic valve near floor with green hose)

- Go outside to valve box under cabin and check valves.

  - Close upper black handle house drain valve by turning it right

  - Close yellow isolation valve

  - Open lower black handle well drain valve by turning left

3.	Position green well drain hose outside of valve box

4.	Start well pump by flipping switch number 6/8 in the bathroom. Water should flow out well drain valve hose, check that the water is clear

5.	If water is clear, close well drain valve and slowly open the yellow isolation valve.

6.	In the cabin the bathroom sink hot water faucet will be spitting with air coming out until the hot water tank fills. When water flows from this hot water faucet, close faucet and start hot water heater by flipping the switch number 26/28 in the bathroom. The water is “ON”.

Close

- Shut off well pump switch by flipping number 6/8 and hot water tank switch by flipping number 26/28 in the bathroom.

- Go outside and close yellow isolation valve in valve box.

- Position green house drain hose outside and open top black handled house drain valve.

- Go into cabin and open all faucets and drain hot water tank by opening gray drain valve near floor connect to green hose. If winter, flush toilet and reservoir and follow number 5 and 7 procedures.

- After the water is drained from the house open blue drain valve under kitchen sink and drain water to pan and discard water.

- Go outside and position green well drain hose outside and open bottom black handled well drain valve. Water should flow out of hose until pressure drops to 20 PSI and then an automatically operated valve opens and suck air into hose to drain water line underground. Water is “OFF”. Open yellow isolation valve to drain any water in line.

- Between mid-October and May, add antifreeze to drains. One cup (8 ozs.) into each sink in kitchen, one in bathroom and TWO in shower drain.  Drain toilet bowl and reservoir of all water using cups and sponge. Add antifreeze to toilet bowl to seal air flow from septic tank.

Pontoon Boat Operation

- Gas instructions. The engine is four cycle; therefore you do not mix gas with oil.  Use regular gas. Gas fill is at the left rear side of boat marked GAS. The tank holds 24 gallons. The gas gauge on the console reads a little less than the actual level in the tank. Add 6-11 gallons of regular gas when the gauge is below half.

- Unlock boat and remove cover. Boat lock key is with boat key. All boat locks are the same.

- Lower motor into water by operating switch on throttle handle.

- Insert key and attach hook on lanyard to kill switch and push switch up.

- Push in neutral button on side of throttle handle and position throttle about half way up.

- Go to gas tank cover and raise cover. A pin on the outside of the cover must be removed to raise cover. Inside the cover locate the gas line bulb. Hand squeeze bulb until hard.

- Start motor by turning key. Push in key to choke motor if cold.

- If the engine is not turning over, the neutral button is not engaged.

- If motor does not start, it may be flooded. Wait a few minutes and start without choking or momentary choking.

- Once motor starts, operate in neutral at about 2500 rpm for five minutes, then bring throttle back to neutral position and allow it to idle until you are ready to leave dock.

- If a hot water alarm light goes off, stop motor and call for service or take it in for engine servicing. Trailer boat to boat dealer.

Closing Boat

- Raise motor out of water by operating the switch on the throttle.

- Check all switches that they are OFF.

- Sweep or vacuum the carpet.

- Wipe the vinyl seats and table. Remove marks with Fantastic and rinse. Additional cleaning instructions are in the Instruction Manual folder.

- Remove all life jackets and dry out storage areas. (This prevents mildew)

- Remove Humminbird depth finder and put in cabin.

- Fold up canopy and cover the boat by using three poles to support cover. Shortest in front, then longest in middle and mid-length in back.

- Lock boat to dock. Locate the tip of the pontoon hole and attach lock through hole in tip to dock chain.

Note: Instruction manuals for boat, motor, radio and Humminbird are in plastic folder in cabinet under TV. Engine: year 2000 50 HP Honda Model BF 50A, serial number 3201085 Spark plug DR7EA by NGK

Boat: 2000 year Bennington Model 2050LX, serial number ETW046371900

Service: Iron Mountain Action Sports, 906-779-1720

Insurance: American Family Mutual call Sue Breitbach-Fenn 920-457-1950

Fishing Boat Operation

- Gas instructions: This is a two cycle engine that mixes its own gas and oil. Use regular gas and pour into the 14 gallon gas tank located at the right rear of the boat. Use Suzuki outboard oil located in the shed and pour into the engine oil reservoir located at the front top of the engine.

- Unlock the boat and uncover. Boat lock key is in cup on boat key chain. Put poles in side compartments. Make sure cover is dry before storing.

- Put keys one red cap and one black cap over cups at back of throttle/shift.

- Lower motor into water by turning black key to on position and operating the switches on the throttle/shift. CAUTION when operating the black key, do not turn the black key too far or you will start engine out of water.

- Lift neutral throttle handle about two inches, located next to throttle/shift

- Pump bulb on gas line until hard

- Turn black key and push in to choke

- Operate engine at high idle with neutral throttle handle for about one or two minutes then use throttle/shift to operate engine.

- Install fish locater in spindle and turn on accessories switch to provide power

- Adjust engine plane down for good operation.

- Add 6 gallons of regular gas at about half of a tank on the gauge

- If the hot water alarm or oil flow alarm light comes on stop using the engine and get service.

- The alarm gauge also has three oil level lights that show the oil level in the engine oil reservoir. Green light shows level and red shows empty. Fill as required.

Closing

- Raise motor and lock the boat to dock

- Shut off all switches; remove depth finder, keys and life preservers.

- Fold all seats and position the upper deck seat with highest point forward to support cover

- Cover boat using two poles. The longest pole is forward.

Note: Instruction manuals in cabinet under TV. Engine is a 1988 75 Hp Suzuki Model DT75TCLJ Serial Number 07501 810462

Boat is a 16 foot LUND.

Trailer EZ Loader ID 1ZE1LJS12GG007876 Year 1986

Shakespeare K350 Kicker electric trolling motor

Service: Falls Action Sports- 5050 Hwy 141 Oconto, WI 920-835-2600

Insurance: American Family Mutual call Sue Breitbach-Fenn 920-457-1950

Wood Burning Stove

Note: This is a Dutchwest Convection Stove by Vermont Castings, Inc Model 7605 The instruction manual is located in the cabinet under the TV.

- The drawing showing the controls is on page 1 of the instruction manual.

- Open the Bypass Damper, Air Inlet Shutters, and Combustor Air Control out one turn.

- Use only uncolored newspaper and dry wood to start fire. Once fire is started add wood and allow temperature to rise to 450°F shown on thermometer on top of stove.

- At or above 450°F close Bypass Damper to burn smoke and allow temperature to rise to 1000°F.

- At or above 1000°F adjust the Air Inlet Shutters to control temperature.

- When adding wood, open Bypass Damper and add wood. Close door and allow the wood about 10-15 minutes to fire up, then close Bypass Damper. Read pages 2 and 3 of the instruction manual for more details of the operation.

- If smoke puffs out from Air Inlet Shutters, open Bypass Damper and allow the wood to fire for 10-15 minutes, and then close Bypass Damper. If smoke continues to puff from the stove the catalytic combustor may need replacement, see manual. This was replaced in 2006.

- If operation of the stove is more that two days empty ash tray.

Cleaning

- Allow the stove to cool and sweep out ashes into ash bucket. Empty ash tray into ash bucket. Ash tray is located below front door of the stove.

- Place ash bucket outside on bricks and or dump in outside fire pit. (Do not dispose around cabin)

- Clean glass on door. Place paper or paper towel under door and spray glass with oven cleaner. Oven cleaner is under kitchen sink in cabinet. Wash glass with all purpose cleaner when done.

- Sweep up floor or wipe floor and outer portion of stove as required.

Parts from Ganon Clay Products in Green Bay located south of 172 on Ashland. 920-498-8549

Cabin Cleanup

- Vacuum cabin, wash shower, clean toilet, sinks and wood stove. Change linens and take dirty linens and towels home and wash.

- Put garbage in garbage can and take to the road. Garbage pickup schedule is located inside kitchen glass cabinet door. Take recycles home.

- Clean up yard and put appropriate items in shed as required. Clean around and cover  fire pit.

- Restock wood under house as required

- Organize and lock shed

- Shut off water, and electricity. (Note, during summer leave refrigerator ON number 1 on circuit breaker panel)

- Close windows and lock cabin and shed.

- Check that boats are locked. (Canoe key is located on key rack in kitchen.)

Operation of Satellite Dish

- Plug in satellite box to electrical outlet

- Plug in white cable to satellite box at the SAT IN location

- Plug in black cable to satellite box at TV/VCR OUT location to TV

- Install card in slot at back of satellite box

- Turn on TV and satellite box. TV needs to be on channel 3

- Use Philips Direct TV controller to change channels.

If there is a problem do not call direct TV contact Ken and Sharon.

Relevant Phone Numbers

House Number: Town of Beecher W9183

Fire and rescue squad: 715-324-5316

Sherriff: 715-735-3309

Electric WI Public Service: 800-450-7240, customer service 800-450-7240

Water Well: Morin & Johnson, Niagra, WI 715-251-1999

Plumber: Wade Schenk 715-759-5965 or Bob Johnke, Wausaukee 715-856-5517

Septic Pumping: Jim Plosczynski, Crivitz, WI 715-854-2205

Firewood: Dick Gelhing at Beaver Creek Lumber Mill 715-324-5471

Richard Rasmussen: 715-324-6288